TECHNICAL SPECIFICATION FOR METAL SPREADING

1.0 SCOPE OF WORK: 1.1 OPTCL is intending to replace the existing and now degraded gravelling in  132KV substation in Odisha to improve the step and touch potential as well as to enable easy and convenient pacing over the gravelling by spreading stone aggregates.

1.2 The Contractor shall provide all labour, equipment and materials required to complete the work in accordance with the drawings, specifications and direction of the Employer (OPTCL) in line with provisional estimate and as per the Instruction of Engineer In-Charge. Procurement of material to be utilized for the said work is  to be done from OPTCL approved vendor (If available).

2.0 General Requirement:

2.1 The material required for site surfacing/stone filling shall be free from all types of organic materials and shall be of standard quality, and as approved by the Employer.

2.2 The material to be used for stone spreading /site surfacing shall be stone aggregate of 40mm nominal size (ungraded single size) conforming to relevant IS standard. Hardness, flakiness shall be as per relevant IS standard.

2.3 The stone aggregates 40mm nominal size may be obtained from the authorized Quarries in the state of Odisha, approved by Quality Control department of Govt. of Odisha with payment of requisite royalty charges.

2.4 The works above shall be done in such a manner so as to cause least or no power interruption. Sectionalizing of areas of the works to be covered in a day may be made and execute them accordingly in consultation with Engineer In charge. Care should be taken so that No step or touch potential should be degraded to the extent for which it possess danger for the operators or the workers in the switchyard while the above mentioned works are done.

2.5 In areas that are considered by the Engineer-in-Charge to be too congested with foundations and structures for proper rolling of the site surfacing material by normal

rolling equipment, the material shall be compacted by hand rammer. Due care shall be exercised so as not to damage any foundation structures or equipment during rolling / compaction.